附錄 A 暫存器表

(Register Map)

AWMF_VER_REG - AWMF Version #00(0x00)

| Field | Bit | Initial Value | Attribute | Description |
| --- | --- | --- | --- | --- |
| AWMF_VER | 31:0 | 0 | R | AWMF Version. (YY/MM/DD) ex: 0x181002(FPGA) → 2018/10/02 |

# CTRL_REG - Control register #01(0x01)

| Field | Bit | Initial Value | Attribute | Description |
| --- | --- | --- | --- | --- |
| CMD_START | 0 | 0 | R/W | SPI command is valid and ready to be executed. ( Need to send a pulse.) |
| CMD_OP | 2:1 | 0 | R/W | 2’b00：STD_WR (Standard Write) 2’b01：STD_RD (Standard Read) 2’b10：AW_BCAST_WR (AWMF Broadcast Write ) 2’b11：AW_CHAIN_RD (AWMF Chain Read) |
| CMD_CPOL | 3 | 0 | R/W | CPOL (Clock Polarity) determines the idle level of the clock. CPOL = 0 means the clock is low when idle. CPOL = 1 means the clock is high when idle. CPHA (Clock Phase) determines on which clock edge data is sampled. CPHA = 0 means data is sampled on the first clock edge (Rising edge when CPOL=0; Falling edge when CPOL=1). CPHA = 1 means data is sampled on the second clock edge (Falling edge when CPOL=0; Rising edge when CPOL=1). |
| CMD_CPHA | 4 | 0 | R/W |  |
|  | 31:5 |  |  | Reserve |

# AWMF_ADDR_REG – AWMF Address register #02(0x02)

| Field | Bit | Initial Value | Attribute | Description |
| --- | --- | --- | --- | --- |
| AWMF_ADDR | 9:0 | 0 | R/W | Specifies the 10-bit address |
|  | 31:10 |  |  | Reserve |

# AWMF_WDATA_L_REG – AWMF Write Data L register #03(0x03)

| Field | Bit | Initial Value | Attribute | Description |
| --- | --- | --- | --- | --- |
| AWMF_DATA | 31:0 | 0 | R/W | Write Data lower 32bit from data out |

# AWMF_WDATA_H_REG – AWMF Write Data H register #04(0x04)

| Field | Bit | Initial Value | Attribute | Description |
| --- | --- | --- | --- | --- |
| AWMF_DATA | 15:0 | 0 | R/W | Write Data High 16bit from data out |
|  | 31:16 |  |  | Reserve |

# AWMF_BC_ADDR_REG – AWMF Write Broadcast ADDR register #05(0x05)

| Field | Bit | Initial Value | Attribute | Description |
| --- | --- | --- | --- | --- |
| AWMF_BC_ADDR | 12:0 | 0 | R/W | Lower 13 bits for broadcast (contains addr10 and other control bits) |
|  | 31:13 |  |  | Reserve |

# SPI_WDATA_L_REG – SPI Write Data L register #06(0x06)

| Field | Bit | Initial Value | Attribute | Description |
| --- | --- | --- | --- | --- |
| AWMF_DATA | 31:0 | 0 | R/W | Write Data lower 32bit from data out |

# SPI_WDATA_H_REG – SPI Write Data H register #07(0x07)

| Field | Bit | Initial Value | Attribute | Description |
| --- | --- | --- | --- | --- |
| AWMF_DATA | 27:0 | 0 | R/W | Write Data High 27bit from data out |
|  | 31:28 |  |  | Reserve |

* Combines the 60-bit standard SPI payload (upper 28 bits from Register 7, lower 32 bits from Register 6).